Tarasafe H2O

Tarasafe H2O is a hardwearing safety flooring collection that comprises of

a PVC wear layer containing deeply inlaid mineral particles and coloured

elements for a subtle and soft design, achieving 1b classification according to the

DSDC.  It is an ideal solution for those areas where enhanced slip resistance is

required for both barefoot and shod use, such as showers, wet rooms, bathrooms, &

changing areas.

The specific embossing, together with the mineral particles, delivers a Class B

barefoot slip resistance rating (according to EN 16165:2021-A and AS 4586-C) and

R11 shod slip resistance rating (according to EN 16165:2021- B and AS 4586-D). It

delivers Safety in Numbers, with a PTV >50 slip resistance (Dry & Wet), a R11 wet

slip resistance rating and >20 Rtm/Rz surface roughness in accordance with HSE

guidelines.

It is reinforced with a glass fibre grid in sheet form for improved durability

The revolutionary SparClean® surface treatment improves the product performance

in terms of stain resistance and durability.

This product is 100% recyclable and contains 29% recycled content.

It does not contain any heavy metals or CMR 1&2 and is 100% compliant

with REACH.

According to ISO 22196, it has anti-bacterial activity against MRSA of 99% after 24h00.

The product emission rate of volatile organic compounds is < 100 µg/m3 (TVOC after 28 days – ISO 16000 -6), which makes it best in class in term of indoor air quality.