Tarasafe Impression

TARASAFE Impression HOP is a stunning looselay, vinyl, safety flooring range that comprises of realistic wood and mineral finishes from Tarasafe Impression range. It is suitable for medium traffic areas, and is ideal for contract applications in healthcare, education, retail and other markets.

The system TARASAFE Impression HOP - floorcovering + holding tape « HOP tape », code 059E0001- enables to lay down flooring glue free.

Thanks to its double fiber frame -fiber veil + fiber grid-, and its calendared interlayer, the floorcovering achieves a high level of stability, even under temperature variations and heavy rolling load.

Without any glue, its installation and end life removal is fast and easy. The Gerflor holding tape doesn’t affect the support, to ease renovation. Re-use of the floorcovering, recycle or renovation are then facilitated.

This product has an improved resistance to moisture. On a new concrete subfloor, which has an integral and effective damp-proof membrane, the relative humidity of the subfloor can be up to 97% RH / 7 CM %.

The Tarasafe Impression Wood and mineral range offers Safety in Numbers, PTV +36 slip resistance, R11 wet slip resistance rating and >20Rtm/Rz surface roughness in accordance with HSE guidelines.

The revolutionary Protecsol Control surface treatment ensures no polish for the entire lifetime of the product.

Tarasafe Impression HOP is a sustainable choice for new build and renovation projects, it contains up to 20% recycled content, can be easily removed, re-used or recycled at end of life.

A multipurpose ﬂoorcovering, available in 2m sheet format with group T wear ratings, ideal for heavy trafﬁc environments. This product comes with a 12-year warranty.